2.7.6/Ovadenco Igor

Part 1

Step 3

How can you verify that both passwords were configured correctly? ---

After exiting user EXEC mode, the switch will prompt for a password to access the console interface and will prompt a second time when accessing the privileged EXEC mode. Also we can use the show run command to view the passwords.

Step 5

Which command do you issue to accomplish this step?---

S1(config)# exit (or end)

S1# copy running-config startup-config

Part 2

Step 2

b. Type the ping command and the IP address for S1 and press Enter.

Packet Tracer PC Command Line 1.0

PC> ping 192.168.1.253

Where you successful? Explain ---

Ping should have been unsuccessful because the switches have not been configured with an IP address.

Part 3

Step 1

If this is the case, why would we configure it with an IP address? ---

In order to connect remotely to a switch, we need to assign it an IP address. The default configuration on the switch is to have the management of the switch controlled through VLAN 1.

Why do you enter the no shutdown command?

The no shutdown command places the interface in an active state.

Step 4

Which command is used to save the configuration file in RAM to NVRAM?

copy running-config startup-config